### Appendix Q

##### Level B Non-Encapsulating

Donning Procedure Checklist

________ 1. All components of the Level B suit ensemble must be gathered and staged

directly next to the member being dressed.

________ 2. All components, including the zipper, of the Level B ensemble shall be

inspected for defects, tears, cuts, abrasions, etc.

________ 3. Establish entry vitals and register the results on the member’s entry

record.

IF THE MEMBER DOES NOT MEET THE VITAL SIGN REQUIREMENTS LISTED ON THE ENTRY RECORD THE MEMBER WILL NOT BE ALLOWED TO DON CHEMICAL PROTECTIVE CLOTHING.

________ 4. Final briefing is conducted by Safety, Entry or Decon Officer.  The briefing

shall review the plan for the member being dressed and all equipment used

to execute the plan shall be checked.

________ 5. Review and have each member perform the emergency hand signals.

________ 6. Have member remove personal items.

________ 7. Have member remove shoes.

________ 8. Assist member in sitting. (Remind member to conserve energy.)

________ 9. Place member’s feet in the suit, one at a time.

________ 10. Place member’s feet in the over boots, one at a time.

________ 11. Don SCBA face piece and perform positive and negative pressure check.

________ 12. Put on gloves:

- Nitrile (Blue) exam glove

- Silver Shield / Ansell glove

- Nitrile (Blue) exam glove

- Green Nitrile glove

________ 13. Assist member in standing.

#### Level B Non-Encapsulating

Donning Procedure (continued)

________ 14. Work suit up and around member and place arms in the suit, one at a time.

________ 15. Place banding over the glove / sleeve closure.

________ 16. Zip the suit up

________ 17. Record the in-suit time on the member’s entry record

________ 18. Put on the member’s hood.

________ 19. Assist member in donning SCBA harness / tank.

________ 20. Assist member in donning protective head gear.

________ 21. Have member perform bend-over test.

________ 22. Perform radio check.

________ 23. Assist member to go on air.

________ 24. Record the on-air time in the member’s entry record.

________ 25. 	Suit Support personnel shall perform a final visual inspection of the suit, checking 		for any damage that may have been caused during donning.

Entry Records are now passed to the Entry or Decon Officer.

The members are ready to perform their assigned task.