Subject:  	Entry Team Group- Supervisor Responsibilities

Purpose:  	Operations during the mitigation of a Hazardous Material Incident should focus on the entry team and their safety.

1	The Entry Team Supervisor shall report to the Haz Mat Branch Director 	(H.M.B.D.).

2	The Entry Team Supervisor shall be a Hazardous Materials Technician.

3	The Entry Team Supervisor shall be identified by a vest.

4 	The Entry Team Supervisor shall assign a minimum of two (2) personnel for

an entry team. Three (3) preferred if manning is available

- The Entry Team Supervisor shall assign a minimum of two (2) personnel for a back-up team.  Three (3) preferred if manning is available.

6 	The entry and back-up team shall be approved by the Medical group prior to entry.

7	The Entry Team Supervisor shall supply one (1) support person per entry team member.

8	The Entry Team Supervisor shall supply the proper protective equipment for each entry team member based on the information received from the Science Information Officer.

9	The Entry Team Supervisor shall brief the entry team and back-up team prior to entry.

10	The Entry Team Supervisor shall monitor air pressure during entry and decon operations of entry team and back-up team members.

11	The Entry Team Supervisor shall communicate with the entry team during  Hot 	Zone operations and continually monitor their status.

12	The Entry Team Supervisor shall debrief the entry team after decon to evaluate progress and additional needs to mitigate the incident.

- Appendices L, N, P and/or Q to be used by the Entry Team Supervisor.