Subject:  Entry And Back - Up Team Responsibilities

- Purpose:  To identify the responsibilities of the entry and back up team.

1	The entry and back up teams shall fall under the control of the Entry Team

Supervisor.

2	Entry and back up team members shall be Hazardous Materials Technicians.

3	Entry and back up team members shall receive a pre-entry check of vitals by Medical group personnel prior to donning chemical protective clothing.

4	Entry and back up team members shall hydrate prior to donning chemical protective

clothing.

5	Entry and back-up team personnel shall place personal belongings in a plastic bag then give the bag to the Entry Team Supervisor.

6	Entry and back up team members shall wear suits providing equal protection.

7	Entry and back up team members shall use SCBA of equal capacity.

8	Entry team and back up team members shall be thoroughly briefed prior to entry into the Hot Zone.

9	The back up team shall be prepared to enter the Hot Zone to rescue or assist the entry team.

10	To prevent heat stress and use of air the back up team shall be partially suited with

chemical protective clothing and off air while on standby.

11	Entry and back up teams exiting the Hot Zone shall proceed to the Decon division for proper decontamination.

12	The entry team shall be debriefed after exiting decon by the Entry Team Supervisor.

13	Entry and back up teams exiting decon shall proceed to Rehab for medical evaluation.