### Appendix P

#### Level A and Level B Encapsulating

# Donning Procedure Checklist

________  1. 	All components of the suit ensemble must be gathered and staged directly next to

the member being dressed.

________  2. 	All components of the suit ensemble, including the zipper, must be inspected for  defects for cuts, tears, abrasions, etc.

________  3. 	Establish entry vitals and register the results on the entry record.

IF ANY MEMBER DOES NOT MEET THE VITAL SIGNS REQUIREMENTS LISTED ON THE ENRTY RECORD, THE MEMBER SHALL NOT BE DRESSED IN CHEMICAL PROTECTIVE CLOTHING.

________  4. 	A final briefing from Safety, Entry, or Decon Division is conducted prior to entry.

The entry team game plan and final check of equipment needed to execute the

plan shall be reviewed.

________  5. 	Review and perform emergency hand signals with all entry team members.

________  6. 	Remove personal items.

________  7. 	Remove the member’s shoes.

________  8. 	Assist the member in sitting. (Remind entrants to conserve energy.)

________  9. 	Place feet in the suit, one at a time.

________  10. Place feet in over boots, one at a time.

________  11. Assist member in standing.

________  12. Don SCBA harness.

________  13. Register tank pressure on the member’s entry record.

________  14. Perform radio check.

________  15. Don SCBA face piece and perform positive and negative pressure check.

#### Level A and B Encalpsulating

Donning Procedure Checklist (continued)

________  16. Don protective head gear and perform bend-over test.

________  17. Put on the inner gloves.

- Nitrile (Blue) exam glove

- Silver Shield / Ansell

- Nitrile (Blue) exam glove

________  18. Work suit up and around SCBA and place arms into suit, one at a time.

________  19. Register in-suit time on the member’s entry record.

________  20. Place on outer gloves and banding. (Kevlar gloves)

________  21. Assist member to go on air.

________  22. Register the on-air time on the member’s entry record.

________  23. Encapsulate member.

________  24. Suit Support personnel shall perform a final visual inspection of the suit, checking     		for any damage that may have been caused by donning the suit.

Entry records shall now be passed to the entry officer.

Team members are now ready to enter the Hot Zone.