Hydrogen Sulfide H2S

A.	INTRODUCTION

Effective implementation of this program requires support from all levels of management.  This program will be communicated to all personnel that are affected by it.  It encompasses the total workplace, regardless of the number of workers employed or the number of work shifts.  It is designed to establish clear goals and objectives.

B.	SCOPE/OVERVIEW

Hydrogen Sulfide is a colorless gas possessing the disagreeable odor associated with rotten eggs.  It is occasionally encountered naturally as the result of decay of organic waste. Sewage and swamp water, for example, typically contain dissolved hydrogen sulfide.  However, in our work environment, the oil and gas industry, it can be found in drilling operations, drilling fluids, tank gauging, field maintenance, and refining operations.

C.	Health effects

Continued inhalation in an atmosphere containing hydrogen sulfide causes dizziness and the onset of a headache.  One deep breath of pure hydrogen sulfide is fatal; breathing a concentration of 600 ppm by volume is fatal within 30 minutes.  Since it possesses such a disagreeable odor, most people are initially aware of its presence.  However, hydrogen sulfide also deadens the sense of smell rapidly.  Thus individuals who remain in an atmosphere containing hydrogen sulfide become oblivious to its presence and may inhale dangerous or lethal amounts unknowingly.  Lack of oxygen to the brain also destroys the phrenic nerve which controls the lungs that control our breathing.  It also causes severe burning and irritation of the eyes.

D.  	OSHA/NIOSH overview

Hydrogen Sulfide is available industrially, mainly as a liquid, in containers.  It is primarily used in the chemical industry to produce other sulfur-containing compounds, but hydrogen sulfide is also used in the metallurgical industry.  In the workplace, OSHA regulates the exposure of employees to hydrogen sulfide.  NIOSH stipulates a permissible exposure limit of 10 ppm by volume of hydrogen sulfide without personal protective equipment.

E.	DOT overview

The Department of Transportation regulates Hydrogen Sulfide as a poisonous gas.  Containers are labeled POISON GAS and FLAMMABLE GAS, and their transport vehicles are similarly placarded.

F. 	NFPA overview

Description:  Colorless gas; offensive strong odor similar to rotten eggs.

Fire and Explosion Hazard:  Flammable gas.  Forms explosive mixtures with air.

Flammable Range:  4.3% and 45%.

Ignition Temperature:  500 degrees.

Vapor Density:  1.2, (vapors are heavier than air) (air = 1.0) will seek lower 	areas.

Boiling Point:  -76 degrees.

Freezing Point:  -117 degrees.

Chemical Abstract Service (CAS) Number:  7783-06-4.

G. 	General requirements

EAGLE INDUSTRIAL INSTRUMENTATION                                               will establish Hydrogen Sulfide operational procedures through the use of this document.

1.  	Facility Evaluation.  This employer shall evaluate our facility(s) or 			host employer facilities to determine if any work area meets the 			criteria for designation as a Hydrogen Sulfide Hazard Area.

H.	  Permit-required confined space program

This employer will implement our confined space program when performing work in areas designated as a confined space.  The permit-required confined space program will conform to the requirements of 29 CFR 1910.146.  This employer shall:

1. 	Implement the measures necessary to prevent unauthorized entry.

2. 		Identify and evaluate the hazards of permit spaces before 					employees enter them.

3. 	Pre-Entry requirements.  Develop and implement the means, 			procedures, and practices necessary for safe permit space 				entry operations, including, but not limited to, the following:

a. 	Specifying acceptable entry conditions.

b. 	Isolating the permit space.

c. 	Purging, inerting, flushing, or ventilating the permit space 			as necessary to eliminate or control atmospheric hazards.

d. 	Provide pedestrian, vehicle, or other barriers as necessary to 		protect entrants from external hazards.

e. 	Verify that conditions in the permit space are acceptable for 		entry throughout the duration of an authorized entry.

f. 	Develop and utilize checklists based on this standard 			practice instruction and 29 CFR 1910.146.

4. 	 Equipment requirements.  Provide the following equipment at no 			cost to 	employees, maintain that equipment properly, and ensure 			that employees are trained in the proper use of the equipment:

- Testing and monitoring equipment needed to determine if 				hazardous conditions exist or to verify that they do not 				exist.

- Ventilating equipment needed to obtain acceptable air 				quality entry conditions.

- Communications equipment necessary for communication 				between personnel involved in the entry operation.

- Personal protective equipment insofar as feasible 					engineering and work practice controls do not adequately 				protect employees.

- Lighting equipment needed to enable employees to see well 			enough to work safely and to exit the space quickly in an 				emergency.

- Barriers and shields as required to protect workers from 				pedestrian and vehicular traffic.

- Ladders, needed for safe ingress and egress by authorized 				entrants.

- Rescue, Retrieval, and Emergency equipment needed to 				extract or treat injured personnel, except to the extent that 				the equipment and or service is provided by rescue services 				that are immediately available.

- Any other equipment necessary for safe entry into and 				rescue from permitted spaces at our facility.

I.	Procedures for atmospheric testing

Atmospheric testing for Hydrogen Sulfide Hazard Areas is required for two 	distinct purposes:  Evaluation of the hazards of the work area and verification 	that acceptable entry conditions for entry into that area exist.

1.  	Evaluation testing.  EAGLE INDUSTRIAL INSTRUMENTATION                           will 	ensure that the atmosphere is analyzed using equipment of 	sufficient sensitivity and specificity to identify and evaluate any 	hazardous atmospheres that may exist or arise.  Evaluation 	and 	interpretation of these data, and development of the entry 	procedure, will be done by, or reviewed by a competent person 	based on evaluation of all serious hazards. The internal 	atmosphere will be tested, with a calibrated 	direct-reading 	instrument, for the following conditions in the order given:

- Oxygen content (19.5% - 23.5%)	OSHA Mandated

- Flammable gases and vapors		OSHA Mandated

- Potential toxic air contaminants  	OSHA Mandated

- Airborne combustible dusts		Site Specific

2.  	Verification testing.  The atmosphere of a work area designated as a permit space which may contain a hazardous atmosphere 	will be tested for residues of all contaminants identified by 	evaluation testing using permit specified equipment to determine that residual concentrations at the time of testing and entry are within the range of acceptable entry conditions. Results of testing (i.e., actual concentration, etc.) will be recorded on the 	permit in the space provided adjacent to the stipulated acceptable entry condition.  The atmosphere will be verified, with a calibrated direct-	reading instrument, for the following conditions in the order given:

- Oxygen content (19.5% - 23.5%)	OSHA Mandated

- Flammable gases and vapors		OSHA Mandated

- Potential toxic air contaminants  	OSHA Mandated

- Airborne combustible dusts		Site Specific

3.  	Duration of testing.  Measurement of values for each atmospheric 			parameter will be made for at least the minimum response 				time of the test instrument specified by the manufacturer.

4. 	Testing stratified atmospheres. When monitoring for entries 			involving a descent 	into atmospheres that may be stratified, the 			atmospheric envelope will be 	tested a distance of approximately 4 			feet (1.22 m) in the direction of travel and to each side.  If a 			sampling probe is used, the entrant's rate of progress will be 			slowed to accommodate the sampling speed and detector response.  		The stratified atmosphere will be tested, with a calibrated direct-			reading instrument, for the following conditions in the order given:

- Oxygen content (19.5% - 23.5%)	OSHA Mandated

- Flammable gases and vapors		OSHA Mandated

- Potential toxic air contaminants  	OSHA Mandated

- Airborne combustible dusts		Site Specific

J.	Training

1. 	Types of training.  EAGLE INDUSTRIAL INSTRUMENTATION will determine whether 	training required for specific jobs will be conducted in a classroom 	or on-the-job.  The degree of training provided shall be determined 	by the complexity of the job and the Hydrogen Sulfide exposure 	hazards associated with the individual job.

a. 	Initial Training.  Prior to job assignment, EAGLE INDUSTRIAL INSTRUMENTATION  shall provide training to ensure that the 	hazards associated with Hydrogen Sulfide are 	understood by employees and that the knowledge, skills and 	personal protective equipment required are acquired by 	employees.  The training shall as a minimum include the 	following:

      - Each authorized employee shall receive training in the recognition of applicable hazards involved with the particular job and job site, as well as the methods and means necessary for safe work.

      - The specific nature of the operation which could result in exposure to Hydrogen Sulfide.

      - The purpose, proper selection, fitting, use and limitation of personal protective equipment (PPE)

      - The adverse health effects associated with excessive exposure to Hydrogen Sulfide.

      - The engineering controls and work practices associated with the employee's job assignment, including training of employees to follow relevant good work practices.

      - The contents of any compliance plan in effect.

b. 	Refresher Training.  Scheduled refresher training will be 		conducted on 	an annual basis.

K.	Retraining

1. 	Retraining shall be provided for all affected employees as a 				minimum under the following conditions:

a. 	Whenever there is a change in job assignments.

b.	 Whenever there is a change in personal protective 				equipment.

c. 	Whenever there is a change in equipment that presents a 			new hazard.

d. 	Whenever there is a change in processes that presents a new 		hazard.

e. 	Whenever their work takes them into hazardous areas.

f. 	Whenever there is a change in Hydrogen Sulfide safety 			procedures.

g. 	Whenever safety procedure fails resulting in a near-miss, 			illness, or injury.

2.  	Additional retraining.  Additional retraining shall also be 				conducted whenever 	a periodic inspection reveals, or whenever 			this employer has reason to 	believe, that there are deviations 			from or inadequacies in the employee's knowledge of known 			hazards, or use of equipment or procedures.

3. 	 The retraining shall reestablish employee proficiency and 				introduce new 	equipment, or revised control methods and 				procedures, as necessary.

4.  	Certification.  EAGLE INDUSTRIAL INSTRUMENTATION                                             shall 			certify that employee 	training has been accomplished and is being 			kept up to date.  The certification shall contain a synopsis of the 			training conducted, each employee's name, and dates of training.

L.	Work operations

1. 	Work operations in which Hydrogen Sulfide may be encountered 			involve welding, burning, cutting, brazing, grinding, and abrasive 			blasting, and general pipe fitting work.

2. 	Employee crew size will vary and employee job responsibilities 			will be that of 	their craft as stated in EAGLE INDUSTRIAL INSTRUMENTATION’S                       		policy manual.  Specific additional responsibilities will be:

a.	 Superintendent/General Supervisor

- Monitors procedure to ensure compliance with this work practice.

b. 	Supervisors

- Ensures that the initial determination for potential Hydrogen Sulfide or toxic exposure has been accomplished before work begins.

- Supervises the safe performance of work in accordance with this and other related work practices.

- Assigns jobs only to qualified employees.

3. 	Employees

- Use the protective and safety equipment as assigned and directed.

- Abide by the requirements of this and site-specific work practices.

M.	MONITORING AND measurement procedures

1. 	Eight Hour Time Weighted Average (TWA) Evaluations.  Where 			possible 8hr TWAs will be taken so that the average eight-hour 			exposure is based on a single eight-hour sample.  Air 				samples will be taken in the employee’s breathing zone and by 			qualified personnel.  The PEC/TWA for acceptable 	and safe 			working environment should not exceed 10 PPM.

2. 	Ceiling Evaluations.  Where possible, measurements to determine 			employee ceiling exposure will be taken during periods of 				maximum expected airborne concentrations of hydrogen Sulfide.  			Each measurement will consist of a fifteen (15) minute 				sample or series of consecutive samples totaling fifteen (15) 			minutes.  Air samples will be taken in the employee’s 				breathing zone and by qualified personnel.

3. 	Peak and Above Ceiling Evaluations.  Measurements to determine 			employee peak exposure will be taken during periods of 				maximum expected airborne 	concentrations of Hydrogen Sulfide.  		Each measurement will consist of a ten (10) minute sample or 			series of consecutive samples totaling ten (10) minutes.  A 				minimum of three measurements will be taken on one work 			shift and the highest of all measurements taken will be assumed to 			be an 	estimate of the employee’s exposure.  Air samples will be 			taken in the employee’s breathing zone and by qualified 				personnel.

4. 	Sampling Methods.  Sampling and analysis will be conducted in 			accordance with acceptable industrial hygiene practices.  				Sampling data will be maintained for the duration of 				employment of the affected employee plus 30 years.

5.  	Whenever alarms sound, stop work immediately and report to pre-			established safe haven areas for roll call.  Never re-enter area 			without proper PPE.

N.	Spill and leak PROCEDURES

Spill and leak procedures will largely depend on the capability and emergency 	procedures of the host employer.  This employer will coordinate with the host 	employer to ensure adequate 	procedures are in-place protection of all employees’ 	(host and contractor) and the surrounding area.

1. 	Persons not wearing protective equipment and clothing will be 			restricted from areas of spills or leaks until cleanup has been 			completed.

2. 	Emergency Containment.  Hydrogen Sulfide exposure can be fatal.  		Only 	authorized and trained emergency response personnel 			should attempt containment.  If you are not trained in 				containment of Hydrogen Sulfide Gas, evacuate the area in 				accordance with established procedures.  If 	Hydrogen Sulfide is 			spilled or leaked the following steps as a minimum 	should be 			taken.

a. 	Remove all ignition sources.

b. 	Ventilate the area of the spill or leak to disperse gas.

c. 	If in gaseous form, stop flow of gas.  If source of leak is a 			cylinder and the leak cannot be stopped in place, remove 			the leaking cylinder to a safe place in the open air and 			repair the leak or allow the cylinder to empty.

d.	 If in a liquid form, allow to vaporize.

O.	Emergency first aid procedures

In the event of an emergency, institute first aid procedures and send for first aid or 	medical assistance in accordance with local procedures.

1.  	Eye Exposure:  Wash immediately with large amounts of water.  			Lifting the lower and upper lids occasionally, get medical 				attention as soon as possible.

2.  	Skin Exposure:  Immediately flush with copious amounts of water.  		Remove any clothing contaminated, and flush exposed skin areas, 			get medical attention as soon as possible.

3.  	Respiratory Exposure: Get the victim to open, fresh air 				immediately.  If breathing has stopped perform CPR.  Keep the 			victim warm and at rest.  Get medical attention as soon as possible.

4.  	Rescue Considerations.  Don’t become a second victim - Put on 			SCBA then move the affected person from the hazardous area.  If 			the exposed person has been overcome, initiate local emergency 			notification procedures. Understand the facility’s emergency rescue 		procedures and know the locations of rescue equipment before the 			need arises.

P.	Protective clothing and personal protective 	equipment (PPE)

Where engineering controls, administrative controls, and job hazard analyses do 	not eliminate all job hazards, employees will (where appropriate) wear 	personal protective equipment (PPE).

1. 	These include items such as caps, hair nets, face shields, safety 			goggles, glasses, hearing protection, foot guards, gloves, 				NIOSH approved supplied air respirator (SAR, SCBA) etc.  			Supervisors will ensure that equipment selected will meet the 			following requirements:

a. 	Any respirator used in an H2S environment MUST be a 			FULL 	face piece mask and be positive  					pressure/pressure demand type.

b. 	It will be appropriate for the particular hazard.

c. 	It will be maintained in good condition.

d.	 It will be properly stored when not in use, to prevent 			damage or loss.

e. 	It will be kept clean, fully functional and sanitary.

2. 	Hazards associated with the wearing of protective clothing, PPE, 			personal clothing and jewelry.  Protective clothing and PPE can 			present additional safety hazards.  Supervisors will ensure that 			workers wear appropriate clothing and PPE.  The following items 			will be monitored so as not to create 	additional hazards.

a. 	Personal clothing and jewelry. Clothing or jewelry that 			could become 	entangled in tools, equipment or machinery 			or of an excessively flammable nature will be prohibited.

3. 	Documentation.  PPE requirements will be documented on a 			“Protective Measures Determination” form and properly filed.

Q.	CONTINGENCY PLANS

These plans should be available and should establish precautionary measures, safety equipment, emergency procedure, responsibilities and duties.

All personnel should become familiar with these plans and sign off that they have read and understand the overall plan.  In addition everyone should also be familiar with all related safety equipment and its use.