| Building comparisons | – PVC membrane seasonal building | Universal architectural membrane permanent bldg. | Pre-engineer Steel building | Brick – block – concrete panel type building | Monolithic Concrete Dome building |
| --- | --- | --- | --- | --- | --- |
| Building removable for summer use to make pool outdoors | YES | NO | NO | NO | NO |
| Building insulation estimate in R-factor | 5 | 10 | 20 | 25 | 60+ |
| Building life span under normal conditions – in years | 12 | 15 | 30 | 40+ | 100+ |
| Cost to insure for replacement | MedHigh | MedHigh | Med | Med | VeryLow |
| Wind resistance – Storm – Tornado - Hurricane | 70 MPH | 70MPH | 90MPH | 90MPH | 150MPH+ |
| Earthquake resistant | Moderate | Moderate | Low | Low | High++ |
| Snow load on roof – tolerances for weight | Low | Low | Med | MedHigh | High++ |
| Ultra Violet deterioration resistant | Low | Low | Med | MedHigh | High++ |
| Cost to operate expressed in $ per square ft. per year. – Utilities rural | $16 | $14 | $12 | $10 | $3 |
| Cost to repair (upkeep) expressed in cents per square ft. per year | $.10 | $.20 | $.50 | $.40 | $.10 |
| Time to construct building | 10 days | 4 weeks | 3-6 mth | 6-8 + mth | 2 mths |
| Is the building leasable | YES | YES | NO | NO | NO |
| Esthetics – scaled on 1-10 (a10 being highest) - (very subjective) | 3 | 4 | 6+ | 9 | 10 |
| Cost to build expressed in $ per square ft. – Rural estimate | $15 | $35 | $145+ | $250++ | $125 |
| HVAC original equipment cost | MedHigh | MedHigh | Med | Med | Low |
| Design cost – fees and services | Low | Low | Med | High | Med |

Some of the things that have to be considered are:

- What is the capital investment limits for the project?

- What are the demographics the facility will serve?

- What are the geographic particulars for your area?

- Is this a phased project or will it be “what you see is what you get”?

- Do you need some pools indoors and some pools outdoors?

- What are your design team’s limitations?

- What are your local code restrictions?

| Type of building |  | 32,000 sq.ft |  | Building | Annual Operational |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cost | Cost - 4 seasons climate |  |  |
| PVC Membrane seasonal building |  |  |  |  |  |  |  |
| convertible to outdoors in summer |  |  |  | $640,000 | $608,000 |  |  |
| Arch.Membrane permanent building |  |  |  | $1,120,000 | $512,000 |  |  |
| Pre-engineered steel building |  |  |  | $4,640,000 | $384,000 |  |  |
| Monolithic Concrete Dome building |  |  |  | $3,900,000 | $224,000 |  |  |

For example:

If you decide on a steel building for the natatorium, the basic operational cost over the first 20 years may be $7.7 million total with a $3.7 million cost to maintain and upgrade over that period of 20 years.  Compare that to a Monolithic Concrete Dome structure which – over the same period of time – should have a basic operational cost of $4.4 million with a $2.2 million cost to maintain and upgrade. Compare $11.4 million to $6.6 million and you be the judge.  Can you afford to build it and operate it?

SERG temporary structure >

Universal arch. membrane structure >

Pre-engineered steel structures >

Brick – concrete – structure >

Monolithic Concrete Dome structure >