| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| David Arenburg Donald Smith Debbie Dauphinee | WAGPI JOHS Committee | MM/DD/YYYY |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Zero Turn Mower | Gas | Steel toe footwear |
| Van With Trailer | Oil | Eye Protection |
| Leaf Blower |  | Ear Protection |

Job Steps

- When operating zero turn in close proximity with fellow employees always make eye contact with them before proceeding. Never come up behind them without their knowing. Likewise the other employees are to make eye contact with the zero turn operator.

- Zero turn refresher required at the beginning of each season. Video or hands on.

- Park van on level surface

- Use safety pylons if parked on side of road

- Undo chain binders on zero turn. Flip safety latch back to allow trailer to drop to unload the zero turn

- Turn on zero turn to half speed, release emergency brake and drive down ramp

- Do hazard assessment of property

- Ensure the grass chute is down, both gas tanks full before proceeding with mowing

- Mow outer limits first

- If area is relatively steep mow across never up and down.

- If area is too steep to use zero turn as tip over is a possibility, use whippersnapper

- Do not mow near vehicles

- Mow with chute away from decks, walkways, driveways and other objects.

- Stop zero turn if there are people in the area or approach the work area.

- Do not allow cut grass to build up on the deck around the spindles/belts. Blow off when warranted

- Turn zero turn speed to half speed and load backwards onto the ramp. Engage parking brake. Turn off.

- Flip safety latch on trailer. Use chain binders to chain zero turn to trailer. Watch out for hot exhaust!

Please note – A hard hat shall be worn when working in close proximity of a building where debris could be thrown and ricocheted.

Please note -Work at schools to be completed only after school hours