| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer Reno Beaver | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Chainsaw | Mixed Gas | Steel toe footwear |
| File | Chain Oil | Chaps |
| Chainsaw Wrench |  | Chain Saw Helmet |
| Wedge |  | Gloves |
| Rake |  |  |

Job Steps

- Check over saw including chain-must be sharp and snug on the bar

- Fill with mixed gas and chain oil if needed.

- Survey tree to be cut and area around the tree.

- Note wind [if any] direction.

- Look for any other obstacles such as power lines, cable lines, Tv lines, buildings, other trees, sidewalks, streets.

- If power lines are within 6m of the tree a NSP Clearance Letter is required before cutting the tree.

- If the area is safe from objects and power lines determine the best area for the tree to be felled.

- Notch tree.

- Remain in constant communication with co-worker as to when tree is being cut.

- Co-worker to stand guard for any possible issues such as people entering the area.

- When all clear cut the tree.

- After the tree has fallen proceed to cut it up per job or customer description.

- Gather up wood and brush

- Place on truck or designated pile

- Rake area.

- Load truck with tools, equipment