| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer Wayne Farrell | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Wheelbarrow | Topsoil | Steel toe footwear |
| Ground Rake | Sod | Gloves |
| Shovel | Stakes |  |
| Knife |  |  |
| Backhoe (Job Specific) |  |  |
| Mallet |  |  |

Job Steps

- Survey area to be sodded. Remove any objects such as large rocks, tree roots sticking up.

- Check for depth and size of area to be sodded. These measurements will assist you in using the right amount of topsoil for the base of the sod.

- Use backhoe [experienced backhoe operator] to spread topsoil on a large area. Use wheelbarrow and ground rakes to fine tune edges or any areas too small for the backhoe.

- Wheelbarrow and ground rakes to spread topsoil on a small area.

- Ideally there should be a sod layer and one who delivers the sod and lays it ahead of the sod layer

- Check for straight run [front house, side of house etc]

- Lay first sod along this straight run.

- As you lay the sod stagger the pieces so that they do not align side by side.

- Keep watch as you are laying to ensure you do not have to cut small pieces of sod.

- Work yourself out from the first piece laid to keep from walking on fresh sod.

- Keep laying until you have reached the end of the row.

- Return and start second row.

- Continue until area is completely sodded

- If sod is on a slope carefully [hands and knees if needed] pound in stakes to hold sod in place until rooted.

- Place fine amount of soil to blend in the ends of the sod to give it finished look

- Gently water sod.