| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer Wayne Farrell | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Stump Grinder | Topsoil | Steel toe footwear |
| Backhoe | Seed | Gloves |
| Ground Rake | Mixed Gas | Eye Protection |
| Fan Rake | Chain Oil | Ear Protection |
| Shovel |  | Chaps |
| Pick |  |  |
| Bar |  |  |
| Chain / Strap |  |  |
| Chainsaw |  |  |

Job Steps

- Perform circle check of backhoe. Add fluids if necessary.

- Do visual check of area ensuring that it is safe. Noting anything above ground that may have underground connections. If unsure, speak to property owner or site manager before digging.

- Using the backhoe [experienced operator] try to dig around the stump to remove it

- If stump will not budge expose more roots and use chainsaw [ use proper chainsaw safety protocols] or axe to cut the roots.

- When stump is removed fill hole with topsoil. Pack down as firmly as possible.

- If stump cannot be removed with a backhoe use the stump grinder.

- The stump should be ground off to at least 2” below ground level.

- Fill with topsoil. Pack down as firmly as possible.

- Before leaving tidy work area and perform circle check picking up all tools and equipment.