| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russel Eisenhauer Wayne Farrell | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Backhoe | 1” Pipe | Steel toe footwear |
| Roller / Tamper | Class A | Gloves |
| Ground Rake | Topsoil | Eye Protection |
| Ground Shovel | Sand | Ear Protection |
| Wheel Barrow | Brick |  |
| Level | 2x4 Lumber |  |
| String |  |  |
| Hammer |  |  |

Job Steps

- Do visual check of area. Remove obstacles

- Measure brick 2.5”-3”, plus 1” sand plus 3”-4” of class A

- Stake area and mark depth.

- Use backhoe to remove excess ground. Level with rake

- Roll or stamp bottom and add 3”- 4” of class a. Roll or stamp again.

- Lay 1” pipes side by side width of walkway and level

- Put layer of sand over pipes. Drag2x4 over pipes to level the sand.

- Carefully remove pipes. Fill holes with sand and hand level.

- Start laying brick when complete. Use a roller or stamper to set brick in place.