| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Dwayne Cunningham Wayne Farrell | WAGPI JOHS Committee | 10/18/2013 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Snow Plow | Pre trip Book | Steel toe footwear |
| Shovel | First Aid Kit | Gloves |
| Tow Rope | Fire Extinguisher | Warm Clothing |
|  | Ice Scraper |  |
|  | Cell Phone |  |
|  | Extra Engine Oil |  |
|  | Extra Transmission Fluid |  |

Job Steps

- Refer to Safe Driving Policy, Driving & Winter Driving Safe Work Practice & Driving Job Procedure.

Refer to Owner’s Manual.  Do not operate machine unless you feel competent.

- Pre-operation check and adjustments

- Conduct a pre-trip inspection of your vehicle.

- Clear ice and snow from your vehicle.

-  Remove items from your pants pockets; they could press on soft tissue and reduce circulation.

- Position items that you may need during driving (e.g. sunglasses) in an easy to reach location.

-  Adjust your mirrors so they can be used without straining your neck or upper body.

- Adjust seat and steering wheel for comfortable driving.

- Warm up your vehicle; it reduces condensation on the windows.

4. Operating the vehicle

-  Keep your two hands on the steering wheel except when shifting gears. Position your hands at “10 and 2 o’clock” on the steering wheel.

- Keep your eyes moving and watch the entire traffic environment.

- Use rearview and side mirrors often.

- Adjust your driving to the road and weather conditions.

- Keep the windshield and windows clear

- For our Fisher & Meyers type snow plows maximum plowing speed is 35km/h.  Maximum transport speed is 60km/hr.  For hydrostatic machines such as the Trackless Sidewak plow, travel in high range & work in low range.

6. Check plow gear for proper operation. (Raise, lower, angle)

7. If all ok, put on seatbelt and put truck in gear.

8.  When entering driveway, do visual inspection of area.  Always push snow away from house, walkway, etc.

9. Make additional pushes as required.

10. Use caution when plowing around lawns, trees, etc as to not destroy.

11.  If you have to plow a driveway, you have not assessed before snow stop, talk to customer if available, and identify hazards.  Make test areas with shovel to locate driveway if unsure of its route.

12.  Avoid backing onto the main road and plowing across the main road.

13.  When finished plowing, park and turn off truck.  Grab a shovel and clear around car, walkways, garage door etc…

14.  Refer to snow shoveling safe work practice.