| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Tony Himmelman Jonathan Langille | WAGPI JOHS Committee | 02/15/2016 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Sand Truck | Sand | Steel toe footwear |
| Snow Shovel | Salt | Warm Clothing |
| Ground Shovel |  | Eye Protection |
| Empty Bucket |  | Gloves |
| Spare Gas |  |  |
| Pipe Wrench |  |  |

Job Steps

- Make yourself familiar with vehicle & spreader. Refer to owner’s manual.

- Refer to SWP for Safe Driving &  Winter Driving

- Pre-trip truck.  Check oil levels in truck and sand spreader.

- Start sand spreader to make sure it is operating correctly & not frozen.  If the spreader is frozen it freed by using a pipe wrench on the shaft or by seeking mechanical assistance.  If you are going to Bridgewater to load sand, use hot water hose at South Shore Sand & Gravel to free spreader.

- Load sand / salt.  If truck is dual rear wheel, capacity is no greater than 2 yards.  If truck has single rear wheel, max capacity is .75 yards.

- Drive to customer location, make sure plowing / snow shoveling is adequate before sanding.

- Turn on flashing lights, start spreader & spread material evenly covering icy areas.  PAY CAUTION TO PEDESTRIANS / PARKED VEHICLES.  DO NOT OPERATE SPREADER IN NEAR VICINITY OF EITHER.

- Do Not Operate Sand Truck with employees riding on the back.

- Using ground shovel & tub / bucket manually sand areas (including walkways) that the truck is unable to reach.

- When done sanding return truck to shop.  Run spreader until all sand is out of the hopper and the chain is clean.

- Clean excess sand off the box of the truck, off the bumper & the taillights.

- Fuel up the truck and the spreader

- Grease the spreader bearings.  If you are unsure of how to do this check with a mechanic / supervisor back at the shop.

- Report any known problems with the truck to mechanics.  Complete a vehicle repair report.