| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer Wayne Farrell | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Stump Grinder | Gravel | Steel toe footwear |
| Ground Shovel | Top Soil | gloves |
| Fan Rake | Seed | Hard hat |
| Gravel Rake | Straight Gas | Ear Protection |
| Chain Saw | Mixed Gas | Eye Protection |
| Whellbarrow | Chain Oil | Chaps |

Job Steps

- Do visual check of the area. Remove any obstacles.

- Cut stump as close to ground as possible with chainsaw.

- Remove all wood pieces.

- Dig around stump with shovel.

- Check over stump grinder before starting. Competent operator only

- Place stump grinder on stump and turn on to grind stump

- Start grinding in a sweeping motion.

- Grind until the stump is fully off.

- Use fan rake to sweep up debris.

- Dispose on back of truck or on a customer designated debris pile

- Wheel gravel or soil [depends on what the customer wants] to hole where the remains of the stump is. Dump in hole.

- Pack down the gravel or soil [ with foot] and keep adding more material until level with ground.

- Fan rake area once again to tidy, gather tools put on truck