| Developed By | Reviewed By |  |
| --- | --- | --- |
| Stephen Oickle Peter Stead Debbie Dauphinee | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Excavator | rock | Safety footwear |
| Dozer | soil | Barricades |
|  | sand | Flagging tape or caution tape |
|  |  | Safety vest |

Job Steps

- The scope of the job will determine which machine is required and if or what type of material is required.

- Grub on the high side of the ditch, leaving a buffer.

- When grubbing on a side slope, you may have to build a bench along the slope to remove the material safely.

- If using trucks to haul, load the grubbing so as not to get stuck in the body or hang out over the sides.

- If dozers are grubbing to a push-off area, always try to push to the sides and leave a buffer at existing tree lines. Do not have the toe of the material on top of any standing trees.

- Ensure any large pieces of wood are broken either by the bucket or the blade.

- When traveling with a dozer or excavator over ungrubbed areas, be sure there are no stakes or stumps protruding which may puncture the bottom of the machine.

- Always prepare a stable access road for haul trucks.

- If grubbing in a low area, it must be piled up. Make sure that there are openings so some water may drain from the area.

- When working around power lines, conform to the written SWP.

- If burying excess material in a hole, make sure it is either backfilled or secured with a barrier and the appropriate signing each night.

- Be aware of any environmental regulations while you are working.

- Ensure any underground wires or services are marked.

- The operator must be aware of any boulders on site. He must be careful not to dislodge them in case they might roll and hit any personnel or equipment on site.