Safe Work Practice

Gas Power Pressure Washer

General – The strong spray from a pressure washer can cause severe injuries and/or property damage. They can also produce a strong kickback. Properly trained employees are the only employees allowed to operate this piece of equipment.

Hazards – Property/ vehicle damage

Severe bodily impact

Hearing damage

Eye injury

Burns

PPE – Steel toed footwear

Hearing protection

Eye protection

Gloves

To start:

-Clean any debris from the inlet filter

-Connect any accessories

-Run water through the washer for one minute to prime the system and remove any air.

-Squeeze the spray wand trigger to bleed water pressure.

-Pull the starter cord to start the engine.

Water Supply:

Make sure your water supply can deliver the gallons per minute specified for your machine. For example, if your pressure washer needs2-1/2 gpm, time how long it takes your garden hose to fill a 5 gallon pail. The garden hose must be 50’long or less and have a ¾” inside diameter, with standard 3/4” hose fittings for connecting to the washer’s inlet. To ensure that water circulates unobstructed through the system, check the water inlet filter or screen and clean it of debris. Also, make sure the garden and pressure hoses are kink free.

Before starting the washer, it is imperative that water is flowing through the washer and out the spray wand. Follow these steps:

-Tighten all hose connections so no air can enter the lines

-set the spray wand to low or no pressure setting to prevent recoil or kickback, when the washer is started. Gas washers with variable nozzles should be on low pressure, wide fan settings. Gas washers with individual nozzle tips should have their nozzle tip removed at this point.

-Completely turn on the water faucet at the house. Squeeze the spray wand trigger to prime the pump and purge air from the system.

-Start the washer. Steady it when pulling the started cord by bracing your foot against a wheel. Let the washer run for a minute to warm up. To avoid damaging the pump: Never run a washer longer than 3-5 minutes [depending on the model] while the trigger is off.