Safe Work Practice

Gas Powered Line Trimmer [Whipper Snipper]

General – A whipper snipper is a high-powered fuel operated piece of machinery commonly used in landscaping. An employee shall not use a whipper snipper without being properly instructed on the operation and safety procedures. A whipper snipper will cause serious personal injury and / or property damage if not used correctly.

Hazards – Noise [hearing damage]

Ejected material [eye damage]

Line

Muffler [burns]

Property/vehicle damage

Injury to public/coworker

PPE – Steel Toed Footwear

Ear Protection

Eye Protection

Protective clothing [pants]

Note: Anti vibration gloves may be worn by employees.

- - Adjust the handle/s to a position for comfort and balance.

- - Machine uses only mixed fuel.

- - Do not fuel machine on pavement or grass.

- - Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

- - When starting, ensure the machine is in a clear area so the line cannot contact the ground or any other obstruction.

- - Switch on choke. Turn on/off switch to on. Pull cord a couple of times until engine starts. Flip switch from choke position to run position.

- - Load the nylon line cutting head only with nylon trimmer line of the proper diameter.

- - String length to be no more than 6”

- - Make sure the muffler side of the engine is away from your body to avoid burns.

- - Maintain a proper balance and secure footing. Do not work on slippery, uneven or unstable surfaces.

- - Maintain a straight wrist position. Avoid using your wrist in a bent, extended or twisted position.

- - Always keep both hands on the control handles. Do not operate one-handed.

- - Keep machine clear of fences, wires, posts, trees, shrubs, foundations to prevent damage.

- - Never whipper snip loose gravel areas

- - Watch for ejected material. Stop trimming to ensure no person or animal is endangered when operating equipment.

- - Stop trimming if vehicles enter work area.

- - Take care when refuelling to avoid spilling fuel on hot motor or exhaust.

- - Shut down immediately if the unit starts to shake or vibrate

- - Do not operate if the line cutter is missing.

- - Do not raise the head above knee height.