| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Tony Himmelman Guy Short John Wentzell | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| String line | Class A | Steel toe footwear |
| Level | Crusher Dust | Eye Protection |
| Gravel Rake | Flagstone |  |
| Pick | Sand |  |
| Ground Shovel |  |  |
| Wheelbarrow |  |  |
| Tamper |  |  |
| Backhoe (Job Specific) |  |  |

Job Steps

- Do a check for obstacles in the proposed walkway area. Ask property owner about any underground wires, pipes etc.

- Proceed to dig out area for walkway. A backhoe may be required for digging depending on the scope of the job.

- Lay class “a” and level

- Tamper class ‘a”

- Lay crusher dust and level

- Lay flagstone [using proper lifting techniques]. If making a walkway to a house start at the door and work yourself “out” to the street, sidewalk etc.

- Ensure each piece of laid flagstone is level. Give it a good stomp with your foot and then check for “rocking” of the rock.

- If need lift the uneven end and put in more crusher dust to stop “rocking” Stomp again. If still rocking remove the flagstone and scratch up crusher dust. Relay the flagstone. Stomp again. May have to use a different piece of flagstone.

- Depending on what the property owner wants-spread sand or crusher dust or soil [ex. may want thyme planted]

- Sweep off the finished walkway.

- Meet with property owner to ensure they are satisfied.

- Clean up debris, tools, equipment.