| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Peter Stead William Gerhardt | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Bucket truck |  | Hard Hatt |
| Pylons |  | Safety Harness |
| Road Signage (Job Specific) |  | Lanyard |
|  |  | Steel Toe Footwear |
|  |  | Reflective Vest |
|  |  | Cell Phone |
|  |  | Hard Hat |

Job Steps

- Complete a hazard assessment at the jobsite.

- Do not operate a boom truck unless you are competent.  You must have fall protection & have training for the specific bucket truck

- Check the truck for manual & fall protection act.

- Locate your harness & lanyard.  Inspect them following instructions in the truck and record your findings.  Do not use a harness or lanyard if it fails inspection.  Stop work until you get a different set.

- Preform circle check of the truck (Using form located in the manual)

- Park on level ground when possible, if need to operate on un-level ground remember your training + refer to manual.

- Apply park brake & chock wheels.

- Put on your harness + lanyard.

- Make sure truck is running, engage pto

- Climb into bucket

- Unstrap the boom

- Secure your lanyard to the anchor point on the truck, ensuring the other end is secured to your harness,

- Begin operation of the truck.  It is a good practice to have a cell phone in your pocket while operating the bucket tuck in case you need to call for assistance.

- When job is completed re strap the boom once it is back to the transport location.  Remember to remove wheel chocks, disengage pto and remove parking brake before driving the bucket truck.