Safe Work Practice

Changing Engine Oil

General- Regular oil changes will improve the engine’s performance, extend its longevity and help your vehicle achieve its peak gas mileage

PPE – Steel toed footwear

Gloves

Eye protection

Hazards – Pinch points

Burns

Rolling

Unbalanced load

Practice - Park vehicle, turn off the engine, set the brake, utilize lock out procedure and place wheel chocks.

2. Place oil pail under the oil pan

3. Remove plug and let drain.

4. Loosen oil filter and remove. Dump contents of the filter into the pail.

5. Clean and check threaded insert and filter base. Ensure sealing ring from used filter is removed.

6. Lubricate sealing ring with clean oil, spin the filter until sealing ring contacts base then turn it once more.

7. Clean oil plug and replace it in the oil pan.

8. Open the hood, remove oil fill cap, place funnel in hole and fill with proper amount of oil, as per manufacturer specifications, then remove the funnel and replace the cap

9. Check oil dipstick and ensure it is at the proper level, if not add more, start engine and let run for several minutes

10. Turn engine off, look under the hood and under vehicle or unit to be sure there are no leaks