Safe Work Practice

Step Ladders

General – To minimize the risk of injuries to employees while working on and around ladders.

Hazards -Ergonomic hazards; force and posture.

-Machine hazards; pinch points.

-Energy hazards; gravity and kinetic.

-Work practice hazards; following established safe work practices and procedures, and general   housekeeping practices.

PPE- Steel toed footwear with heals and slip resistant soles

-Safety glasses

-Other job specific PPE

Safe Work Practices

Read and follow the manufacturer’s instructions and warning labels.

Know how to properly inspect and safely erect a portable ladder.

Commercially manufactured portable ladders must be CSA approved.

Only one person is permitted to be on a ladder at any time.

Ensure that all electrical equipment that will be used during ladder work is in good working condition and is properly grounded.

Ensure the work area is clear of debris.

Ensure there is adequate lighting in the work area.

Only use a stepladder for light duty tasks of short duration (less than 30 minutes) where your centre of gravity is maintained between the side rails.

Do not use a ladder if you feel drowsy or unwell.

Do not overreach while on a ladder. Move a stepladder when needed.

Do not stand, climb or sit on the top of a stepladder.

Do not "shift" or "walk" a stepladder when standing on it.

Do not use a stepladder as a brace or support for a work platform or plank. Do not climb a stepladder that is leaning against a wall. Use a straight ladder instead.

Do not use a stepladder on a slippery surface.

Do not use a stepladder on soft ground; the legs could sink into the ground.

Do not place a stepladder on a box, unstable base or on a scaffold to gain additional height.

Do not climb the back of a stepladder.

Do not carry objects in your hands while climbing a ladder. Hoist materials or use a tool belt.

Do not paint a wooden ladder.

Do not try to straighten or attempt to use a bent or bowed ladder.

Do not straddle the space between a ladder and another object/surface.

Do not push or pull a stepladder sideways. Repeated sideways movement can make a stepladder wobbly.

Do not use a ladder in a passageway, doorway, driveway or other location where a person or vehicle could hit it. Set up suitable barriers or lock door