| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Lawson Fraser Jack Durnford | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Chainsaw | Fire Starter | Steel toe footwear |
| Rakes | Lighter | Chainsaw Helmet |
| Shovels |  | Chaps |
| Water Source |  | Gloves |
| Fire Extinguisher |  |  |

Job Steps

- Follow Municipal guidelines [permit, burning restrictions]

- Monitor weather-i.e wind – too windy do not burn

- Establish a safety ring [dig circle, remove burnable material] around the brush pile to contain any possible fire spread.

- Ensure your water source/fire extinguisher is close to the fire in case of an emergency.

- Put smaller branches/cardboard on pile to start fire

- Gradually build the fire by adding small branches

- Once the fire is established, add larger branches by crossing them onto one another

- Never leave the fire unattended and never allow it too become too high

- Leave enough time at end of day to allow fire to die down

- Extinguish the fire, sifting embers and adding more water until no more hot spots