| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| John Wentzell Person 2 Person 3 | WAGPI JOHS Committee | 9/13/22 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Utility Knife | Gyproc | Steel toe footwear |
| Measuring Tape | Gyproc Screws | Eye protection |
| Pencil |  | Mask |
| Drywall Knife |  |  |
| Step Ladder |  |  |
| Screw Gun |  |  |

Job Steps

- Do a hazard assessment checking for wires, pipes etc

- Mark position of studs at top and bottom of wall

- Cut gyprock to fit [floor to ceiling and stud to stud

- Hold against wall

- Screw to studs with gyprock screws using screw gun

- Cut out holes for wall plugs/switches etc prior to screwing to wall

- If ceiling is to be drywalled, do ceiling first, install wall tight to ceiling.

- Clean up