Safe Work Practice

Vibrating Plate Tamper

General

A plate tamper is a fuel powered heavy piece of equipment vital to the compaction of soil and or rock. Crushing injuries may occur from inadequate training or lack of experience with the machine.

Hazards

Noise

Vibrations

Exhaust fumes

Musculoskeletal problems

Hot Engine

PPE

Steel toed footwear

Hearing protection

Eye protection

Dust mask

Gloves [anti vibration]

Safe Operating Procedure

Note: Must use assistance to lift the tamper. Practice safe lifting techniques

- Do hazard assessment for any potential obstacles

- Check compactors fuel level, oil level, air filter

- Turn fuel on (lever down)

- Close choke and place throttle at idle

- Pull rope start in a smooth action, keeping knees bent and back straight

- Once started, open choke and allow to warm up

- Move throttle quickly to full position

- Let compactor plane along by itself, using the handles to steer the machine along

- Bring the machine back to idle to stay stationary or when travelling over compacted or solid ground.

- Work progressively over the area to be compacted, to prevent the machine from ‘digging in’

- Spray area with water to suppress dust and to aid with compaction

- The area should be compacted 3 to 4x

When finished:

- Allow compactor to idle for a few seconds before pressing the stope button

- Ensure the stop button is depressed until the motor is completely stopped

- Once the motor has cooled the machine can be cleaned.