| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Tony Himmelman Jonathan Langille | WAGPI JOHS Committee | 10/11/2013 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Backhoe | Culvert | Steel toe footwear |
| Shovel | Class A | Ear Protection |
| Rake | Class C | Eye Protection |
| Pylons | Wall Rock | Gloves |
| Barricades |  | Hard Hat |
| Signage |  |  |

Job Steps

- Review Safe Work Practice, Choose competent employees, determine permits required & culvert specs

- Preform hazard assessment, i.e. overhead/underground wires

- Check over backhoe

- Dig trench with downward flow

- Put culvert in place

- Use wall rock / other large rock to buil wall at each end of culvert holding material in place

- Cover culvert with shale

- Cover shale with class A.

- Drive over Class A with backhoe to compact.

- Perform environmental inspection