| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Russell Eisenhauer | WAGPI JOHS Committee | 11/19/2018 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Hedge Trimmer |  | Eye Protection |
| Fuel |  | Ear Protection |
| Ladder |  | Gloves |
|  |  | Steel Toe Footwear |

General

A gas powered hedge trimmer is a commonly used tool in landscaping. It can cause serious injury if employees are not properly trained in its’ usage.

Hazards

- Exposure to noise

- Entanglement

- Burns

- Cuts

- Slips, trips, falls

- Manual handling injuries

Job Steps

- Hazard Assessment. Inspect the hedge for any obstacles. Look for wire, pipes, cables etc. Remove or flag obstacles.

- Visually inspect hedge trimmer before use. Check that the trimmer is in good operating order. Make sure blades are sharp, lubricated and move freely.  Check the motor runs smoothly, the auto-switch mechanism is working correctly and that safety guards are in place.

- Hold trimmer firmly in both hands.

- Ensure firm footing

- Move your body with the trimmer; do not twist from the waist.

- Do not cut over shoulder height.

- Do not walk with trimmer whilst cutter is spinning.

- Turn off trimmer prior to refueling.