| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| John Wentzell Tony Himmelman | WAGPI JOHS Committee | 04/18/2018 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Gas Drill | Wharf Ties | Steel toe footwear |
| Jack Hammer | Timbers | Eye protection |
| Wrenches | Decking | Ear protection |
| Chainsaw | Crib Rock | Gloves |
| Backhoe | Bolts | Life Jacket |
| Generator | Spikes |  |
|  | Lumber |  |
|  | concrete |  |

Job Steps

- Tidal times will determine when and length of time the wharf can be worked on

- Anchor and set vertical corner posts [crib]

- Drill and bolt horizontal ties [crib]

- Fill crib with rock

- Form and pour concrete for pads, ramps, footings

- Position and fasten wharf beams

- Install wharf decking and bracing

- Position, anchor and install support beams if needed

- Install kick rails etc

- Clean up debris and unused material

- Meet with customer at job completion