| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| William Gerhardt Daniel Stead | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Water Truck | Gas | Steel toe footwear |
| Hoses |  | Life jacket |
| Pump |  |  |

Job Steps

- Pretrip truck and inspect and ensure tank is loaded and secure.

- Fill water tank before leaving for delivery

- Get delivery instructions -what is being filled and address with any special instructions

- Make sure you have adequate length of hose

- Ensure pump works and is secure, hoses secure, valves closed.

- Drive / follow all Motor vehicle regulations to location

- Park on level [ watching for soft areas and/ or septic] and as close as possible to unloading point. Apply all brakes before exiting truck and in a safe place

- Do hazard assessment of unloading place

- Use any applicable safety equipment

- Run hose

- Begin pumping by turning on the pump and opening the valves

- When finished, close valves, turn off pump, secure pump and hose.

- Return to shop and refill tank for next delivery [unless it is below freezing then do not refill]