| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Person 1 Person 2 Person 3 | WAGPI JOHS Committee | MM/DD/YYYY |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Ground Shovel | Topsoil | Steel toe Footwear |
| Tiller | Compost | Ear Protection |
| Edger | Landscape Fabric | Eye Protection |
| Ground Rake | Decorative Rock or Mulch |  |
| Fan Rake |  |  |
| Wheelbarrow |  |  |
| Tarp |  |  |
| Marking Material |  |  |

Job Steps

- Unload truck and place near work area

- Do hazard assessment looking for any above or underground obstacles

- Meet with customer to get design of garden. Layout design using marking material

- Depending on size of garden use the tiller or rd pt shovel to dig up the area.

- Rake of larger chunks of turf. Dump on truck or customer designated debris pile.

- Use edger to define the edge of the garden

- Add topsoil, compost

- Pull back the topsoil, compost to make a distinct raised edge

- If required by customer top dress with mulch and or decorative stone. Carefully keeping it away from the edge of the turf

- Using fan rake, rake the turf bordering the garden to give it a finished look.

- Meet with customer before e leaving

- Load tools and secure load

- Complete timesheet