| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer | WAGPI JOHS Committee | 10/11/2013 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Chains |  | Steel toe footwear |
| Straps |  | Hard Hat |
| Binders |  | Eye Protection |
| Truck |  | Gloves |
| Directions |  | Safety Vest |

Job Steps

- Pre trip truck

- Get directions, name, detail, phone number and description of what you are picking up.

- Preform Hazard assessment when arriving on scene, check equipment you will be hauling to determine safest way to move and load.

- Check compliance with road rules, i.e. height, width, weight.

- Back into equipment and lower deck.

- Extend cable and fasten to equipment.  Winch equipment onto deck.

- Secure back of equipment from moving ahead to prevent movement while raising the deck.

- Raise the deck and finish securing load.

- For safety stop and check the load after driving a couple of miles.