Safe Work Practice                                                                                                                                                  Updated Nov.15/18

Use of Tiger Torches

General – Tiger torches, although valuable to a job site, are sometimes misused in a manner that can make them dangerous.

Hazards – compressed gas

burns

fire/ explosion

inhalation of hazardous fumes or vapours

repetitive strain

PPE-steel toed footwear

eye protection

hand protection

flame retardant clothing

fire extinguisher

Tiger torches are only to be used for pre heating of piping etc prior to welding.

Do not use a tiger torch to heat a propane tank.

Practices

1.Only qualified personnel to use the torches.

2.When a torch is used, an adequate fire extinguisher should be present

3.Eye protection must be worn when heating edges where loose aggregate is present, because the rocks can explode due to the extreme heat that is applied to them.

4.Ensure fuel lines are in good working conditions. Inspect the hoses and valves to ensure there are no leaks.  If you suspect that there is a leak, do not use a match to test.  Instead, use soap and water and look for bubbles.

5.Ensure proper cylinders are secured and regulators in place

6.Ensure that the propane bottle is in the upright position during use of the torch.

7.Follow proper procedures for lighting torch.

8.When not used for pre-heating operation, shut torch off. Do not leave the tiger torch on, unattended.

9.Torches are not to be used for heating or thawing of lines where known hydrocarbons are present.

10.Ensure that the propane bottles are properly shut off.

11.Ensure that you turn the propane off at the tank and not just the tiger torch head.  This will ensure that no propane will leak out of the tank if there is a leak in the propane line linking the tank to the torch.