| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Dwayne Cunningham Debbie Dauphinee Randy Naugler | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Welder | Metal | Welding Helmet |
|  |  | Welding Gloves |
|  |  | Steel toe footwear |
|  |  | Flame retardant clothing |

Job Steps

Welding presents several hazards to the welder and for others who may be in the vicinity. Therefore it is important to understand the hazards associated with welding.

Hazards

- Electrical risks

- Heat related risks

- Fire and explosion

- Fumes

- Gas use / storage

- Health related risks – including asphyxiation, respiratory issues cancers, damage to the nervous system

Job Steps

- Ensure the machine is correctly set up for current, voltage, wire feed and shielding gas flow rate.

- Ensure that other workers in this locality are protected from any UV & IR radiation flash. Always close the UV curtain to the welding bay or erect a UV screen.

- Ensure the welding work cable (earth) makes firm contact to provide a good electrical contact.

- 4 Ensure the work piece has been prepared to be free of any paint, oxides or other surface finishes ensuring a good electrical contact.

- Take particular care to avoid accidental UV welding flash to the skin or eyes.

- Never leave the MIG welder powered on unattended

- Regularly inspect the welding tip and shield for damage.

- When welding is finished or interrupted, turn off the shielding gas at the regulator, turn off the machine and secure the handpiece safely.

- Leave the work area and bench in safe, clean and tidy condition

- Never leave hot work unattended