| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Gretchen Gerhardt Darren Haughn | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Tiller | Gas | Steel toe footwear |
| Planks / Ramps | Oil | Safety glasses |
|  |  | Gloves |

Job Steps

- Get planks from truck.

- Push brake on tiller to neutral. Push tiller slowly down the planks.

- When tiller reaches the ground, put brake back on. Check the oil and gas – add if needed.

- Push brake to neutral and push it to area that needs to be tilled.

- Survey area. Remove any large rocks or items that would damage the tiller

- Put throttle to choke.

- Put gear shift in neutral.

- Pull starter cord – keep pulling cord until tiller starts.

- Stand behind tiller. Check to ensure safety plate in rear of rungs is in place.

- Put right hand on right handle. With left hand push rotor engagement lever [between handles] down. Move gear shift to slow. Begin walking with tiller. Put left hand on left handle.

- Push down on the back of the tiller. Moving straight forward. Do not at any time remove hands from handles or put feet near rotors.

- When near perimeter of area being tilled, lift up on handle for rotor engagement and quickly swing handles around. Put lever back down and begin tilling again.

- When area has been tilled once, stop the tiller and remove large rocks from area.

- Start tiller again using the same sequence. Alter the direction you tilled the rows.

- When tilling is finished, turn off tiller, put in neutral and push to the truck. Do not touch the muffler on the tiller- extremely hot.

- Push tiller onto the truck the same way it was removed. Put tiller in place. Tie it in place