| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Gretchen Gerhardt Darren Haughn Debbie Dauphinee | WAGPI JOHS Committee | 05/25/2018 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Push Mower | Oil | Steel toe footwear |
|  | Gas | Eye Protection |
|  |  | Ear Protection |

Job Steps

- If 2 people present, lift on opposite sides of the mower base and remove mower from van. Practice safe lifting techniques. Lift with your knees. If only 1 person present place one hand on the front base of the mower and the other hand on the back and remove mower from van . Practice safe lifting techniques.  Lift with your knees.

- Check the mower for full gas tank. Do not fill gas on the lawn.

- Ensure protective flap is in proper place [grass chute]

- Walk over area to be mowed noting any hazards [ hazard assessment] and correcting if possible

- Put mower on lawn making sure the chute is not aimed at objects

- Push primer button 3x

- Hold down safety bar located at top of handle

- Pull back on the pull cord.

- When the mower starts, start mowing forward. Never “shoot” the grass at any object or people

- If people approach your area stop mowing until they have left the area.

- While mowing, be constantly watching for any type of obstacle that may have appeared since starting the lawn. Conditions can change quickly.

- Slightly over lap when cutting the grass as to not leave strips of uncut grass.

- If grass chute becomes plugged, turn off mower and remove clogged grass. NEVER put your hands near the chute with mower running

- When mowing in close proximity with other employees always make eye contact with the other operator first before approaching

- If the area is relatively steep, mow across the hill and not up and down. If the area is too steep to mow then use a whippersnipper.

*To test if an area is too steep -turn off mower and place it sideways on the bank. Let go of handle but keep hands nearby. If the mower tips then it is too steep to use push mower

- When finished mowing, release hands from safety bar. Take mower to van

- 2 people to lift mower into van using safe lifting practices. Use knees to lift and watch out for hot exhaust/engine

- If lifting mower alone follow same procedure as getting the mower out of van and watching out for hot exhaust/engine

- Blow off any grass in the walkways, driveways or any other objects

Please note – Any work to be completed at schools will be done after school hours

Please note- A hard hat shall be worn if mowing in close proximity of buildings or any area where debris could be thrown and ricocheted.