| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Russell Eisenhauer Wayne Farrell | WAGPI JOHS Committee | 04/30/1999 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Backhoe |  | Steel toe footwear |
| Jack Hammer |  | Gloves |
| Sledge Hammer |  | Eye Protection |
| Hacksaw |  | Ear Protection |
| Pry bar |  | Hard Hat |
| Chain/Strap |  |  |
| Shovel |  |  |
| Pick |  |  |
| Rake |  |  |

Job Steps

- Operator of backhoe to be competent and have reviewed operator manual prior to use

- Do visual check of area. Remove all hazards. Block work area off from public.

- Use backhoe to dig around cement wall.

- Do not stand below wall as it may shift while you are working.

- Brace the wall if there as any concern that it may shift. Use 2x4 lumber.

- Use jackhammer [experienced operator only] to break up cement.

- Remove cement with backhoe or shovel.

- Continue this practice until all the cement is broken up and removed.

- A sledgehammer may be used near the completion to break the cement.

- Reminder to never lift more than you are comfortable and to use good manual lifting techniques.

- At the end of the job clean up the work area