| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Daniel Stead Peter Stead | WAGPI JOHS Committee | MM/DD/YYYY |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Wheel Chocks |  | Steel toe footwear |
| Jacks |  | Gloves |
| Impact Gun |  | Eye Protection |
| Jack Stands |  |  |

Preparation

1. Set parking brake and block wheels.

2. Gather all necessary equipment and tools.

3. Completely deflate tire to be changed.

4. Ensure good lighting.

Job Steps

1. Place hardwood blocks under the jack. Erect safety stands under the vehicle.

2. To avoid loss of control injuries, keep a firm grip on tire irons and other tools.

3. Jack up the vehicle so the tire is off the ground, allowing sufficient space to place the tire onto the rim. 
4. Loosen lug nuts but do not remove.

5. Hit tire with mallet to break loose.

6. Remove lug nuts and tire.

7. Install new tire and lug nuts if needed.

8. Make sure valve stems are 180 degrees away from each other for each side and line up for access.

9. Tighten lug nuts evenly with a few turns alternating between all nuts until tight

10. Lower the vehicle with the jack until all the weight is off the jack and remove.

11. Remove blocks and release parking brake. Remove lock.

12. After one trip, check to make sure lug nuts have not become loose.

Please note the above procedure is for SPOKE WHEELS

For BUD WHEELS lower the vehicle first, release brake and then tighten lug nuts.