| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Daniel Stead Peter Stead | WAGPI JOHS Committee | MM/DD/YYYY |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Wheel Chocks |  | Steel toe footwear |
| Jack |  | Eye protection |
| Jack Stands |  | gloves |
| Impact Gun |  |  |
| Socket |  |  |

Preparation

1. Set parking brake and block wheels.

2. Gather all necessary equipment and tools.

3. Ensure good lighting.

Job Steps

1. If equipped with hubcaps, remove with tire iron.

2. Place sound wood blocks under the jack. Erect safety stands under the vehicle.

3. To avoid loss of control injuries, keep a firm grip on tire irons and other tools.

4. Jack up the vehicle so tire is off the ground, allowing sufficient space to place the tire onto the rim.

5. Loosen lug nuts but do not remove.

6. Hit tire with mallet to break loose.

7. Remove lug nuts and tire.

8. Install new tire and lug nuts if needed.

9. Tighten lug nuts evenly with a few turns alternating between all nuts until tight.

10. Lower vehicle with the jack until all the weight is off the jack and remove.

11. Remove blocks and release parking brake. Remove lock.

12. After one trip, check to make sure lug nuts have not become loose.

Please note that the above procedure are for SPOKE WHEELS

For BUD WHEELS    lower the vehicle first, release brake and then tighten lug nuts.