| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Bill Corkum | WAGPI JOHS Committee | 09/20/2022 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Dump Truck |  | Steel Toe Boots |
|  |  | Safety Vest |
|  |  | Hard Hat(in truck) |

Job Steps

- The safe operation of company vehicles is the responsibility of the operator who will abide by the owner’s manual of the vehicle, provincial laws and company procedures.

- Every employee shall be properly licensed and is required to always have their license in their possession when driving a company vehicle.

- Perform a pre-trip of the truck as per standard company log sheet including the tailgate latches to make sure they are secure. [log sheet attached]. If any additional questions arise consult the owner’s manual.

- Minor maintenance tasks including but not limited to [vehicle cleanliness, greasing, tire pressure and brake adjustments] are completed according to the provincial laws, owner’s manual and company procedures.

- Any issues arising from the pre-trip must be addressed before leaving the yard.  If, this cannot be done the vehicle is put out of service until the problem is corrected.

- Write the starting mileage on the timesheet to begin the log.

- Start the truck. Adjust mirrors. Once again checking to make sure nothing is hanging off of the truck

- The vehicle is to be driven according to road conditions, load, and speed limits. Dump vehicles are not designed for speed and can easily tip on corners especially when the vehicle is loaded. Seatbelts are to be always worn

- Avoid soft ground. If it is a new site, walk the site before backing or driving on it to check for large holes, overhead obstacles, or wet areas.

- If there is working machinery, make the operators aware of you by:

- making eye contact

- sound the truck horn

- waiting for the operator to signal you to approach.

- use of phone [if available]

- never approach the equipment if the cab is not facing in your direction.

- Loading of truck-Park on level ground.

- Load should be distributed evenly in the box.

- Do not overload.

- Make sure no part of the load is hanging off or on the sides of the truck

- Tarp the load

- Dumping of load-Must be on level ground

- Check for overhead obstacles such as trees or wires etc

- If working around wires use a “spotter.”

- Remove tarp.

- Make sure tailgate is unlatched.

- Engage power take-off.

- Put dump lever into lift position.

- Dump load.

- If load sticks, move ahead slowly to dislodge the remainder of the load. The vehicle should only move marginally ahead to unload.

- Put vehicle in park and driver should go to the back of the vehicle to make sure the entire load has been dumped.

- Put the box down and latch the tail gate.

- If dumping in a muddy place-after dumping-pull out of mud area and    clean out any debris lodged between the tires

- If dumping in a rocky place-after dumping-pull out of rocky area and remove any rocks stuck between the tires as they will cause a flat tire

- Once box is down, make sure the latches are secure and there isn’t any debris caught in the tailgate or on the sides of the truck.

- At the end of the day, the vehicle is to be fueled up and backed in a parking spot. Depending on weather conditions the box may be left in the raised position.

- Finish the timesheet/log at the end of the day documenting any minor maintenance issues that may have come up during the day.