| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Stephen Baker Peter Stead | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Sledge Hammer | 2x4 or 2x4 for stakes | Steel toe footwear |
| Claw Hammer | Lathes | Gloves |
| Crow Bar | Snow Fencing | Eye Protection |
|  | Lath Nails |  |

Job Steps

- Set first post in ground, will probably require hole made with crow bar, drive home with sledge hammer.

- Weave lath through fencing top to bottom.

- Nail lath to post, top, bottom and mid point, do not puncture fence.

- Repeat steps 1 – 3 at 10’ intervals stretching the fencing tight between posts.