| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Peter Stead | WAGPI JOHS Committee | 11/04/2018 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Truck / Machinery to Fuel | Fuel | Steel toe Footwear |

General

- Fueling of equipment / vehicles is a daily task which can be hazardous if not done properly.

- Cell phones may not be used while you are fueling or used near the fuel cage while someone else is fueling.  Smoking is prohibited at all times near the fuel cage.

- In the event of a minor spill there is a spill kit located inside the back door of the shop to the left hand side when entering the shop.

Hazard

- Cell phones

- Smoking

- Risk of static shock / fire.  Never fuel a plastic gas jug while it is on the truck with a plastic liner.  The jug may have a static charge from sliding on the liner. In the event of fire back away, have office shut off power to pump.

Job Steps

- Be familiar with the equipment you are fueling.  Refer to the manufacturers procedure for fueling.

- Determine if your equipment runs on straight gas, mixed gas or diesel.

- Report to the office, turn on power to the pumps and take the fuel book with key.

- Unlock fuel cage and begin your recording.

- Place appropriate nozzle into equipment fuel tank and start the pump.

- Begin fueling, avoid spilling fuel on the equipment or ground.  If a leak is spotted turn off the pump and report to maintenance staff inside the shop.

- When fueling complete turn off pump, remove nozzle and coil fuel hose that no part of the hose is left outside the fuel cage.

- Finish recording, lock fuel cage. Recording requires all columns to be completed.

- Return fuel book to office and shut off power to pumps.