| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Peter Stead | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Wheelbarrow | Mulch | Steel toe footwear |
| Ground Shovel |  | Gloves |
| Flat Shovel |  | Eye Protection |
| Ground Rake |  | Ear Protection |
| Fan Rake |  |  |
| Bucket |  |  |
| Tarp |  |  |
| Broom |  |  |
| Leaf Blower |  |  |
| Mixed Gas |  |  |

Job Steps

- Unload tools from truck using safe lifting techniques

- Complete hazard assessment.

- Put tarp under tailgate to catch mulch to minimize clean up

- If dumping the load of mulch from 1 ton / larger truck put tarp down and unload on tarp.

- A new garden should have a layer of mulch 2.5”-3” deep.  If top dressing [already mulched] garden .5”-1” is sufficient.

- Start at the furthest garden and work towards the mulch pile

- Start at the back of the garden and work yourself out leaving no footprints.

- Lifting branches, leaves, flowers as necessary and put mulch underneath.

- Move garden décor then mulch.

- May be necessary due to plants/shrubs, small space or access to use buckets to carry mulch.

- Spread mulch with hard rake or hands depending on size and openness of garden.

- If using hands and are required to get close to shrubs to spread mulch be careful when moving branches as the branches can fling mulch into your eyes.

- Do not “volcano” mound trees and shrubs. This invites pests and rotting of the tree /shrub

- Do not mulch right up to the prepared edge of garden. Leave a small gully to serve as a weed stopper. This also gives the garden a “finished” look.

- Put back all garden décor.

- Fan rake the edges of the lawn where it meets the garden to clean any spilled mulch and refresh grass.

- Clean the area around the truck where mulch was dumped using broom and flat shovel.

- Blow remaining mulch debris from driveway /walkways/decks