| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Darren Haughn Gretchen Gerhardt Debbie Dauphinee | WAGPI JOHS Committee | 12/29/2021 05/25/2018 09/27/2005 10/01/2000 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Whipper Snipper | Trimmer Twine | Safety Boots |
| Leaf Blower | Mixed Gas | Work Pants |
|  |  | Ear Protection |
|  |  | Eye Protection |
|  |  | Gloves |
|  |  | Hart Hat – when conditions warrant |
|  |  | High Visibility Clothing – when conditions warrant |

Job Steps

- Operator must be competent, trained and have reviewed the products owner manual.

- Ensure you are wearing proper PPE for the task and your surroundings. When working in close proximity of mower or any area where debris can be thrown or ricocheted, a hard hat shall be worn

- Visual inspection of machine, check for damage and ensure guard is in place

- Check string length. Set at 6”

- Fill machine with mixed gas. Fill at each new site in an open well-ventilated area off pavement/concrete

- Turn kill switch on, put machine on choke

- Pull cord until machine starts, pushing throttle will deactivate choke.

- Carefully turn machine around bushes,trees [ never hitting the shrubs /trees as this will eventually kill them]

- Carefully work around foundations, walls, etc. Go as close as possible without striking.

- Keep your left-hand side away from gardens, driveways & other objects.  The whipper snipper will throw objects to the left.

- Never whipper snip loose gravel

10.Do not go close to vehicles or people.

11. If people approach your work area stop whipper snipping until they leave the area.

12. When working in close proximity with zero turn mower DO NOT APPROACH ZERO TURN without making eye contact with operator first

13.When job is finished, turn machine off-do visual check to ensure no area has been missed.

13. Blow off any grass on driveways, walkways/ objects

14. Return the whipper snipper to the van in proper place.

Notes

- All work to be completed at any schools to be done after school hours.

- A hard hat shall be worn when working in proximity of buildings or any area where debris could be thrown and ricocheted.

- This Job Procedure has been developed for Stihl Trimmers/Brush Cutters used at William A. Gerhardt Property Improvement.  This Job Procedure may not be accurate for other brands.

- To avoid wrist injury anti-vibration gloves are recommended.