| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Wayne Farrell | WAGPI JOHS Committee | 04/18/2018 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Roller |  | Ear Protection |
|  |  | Steel toe Footwear |
|  |  | Safety Vest |
|  |  | Hard Hat (Site Specific) |

General

Employees who operate or work around rollers / compactors are at risk for injury from a machine roll over or being struck by the machine or its components. The operator must be trained and have experience operating a ride on roller.

Hazards

- Severe bodily impact

- Musculoskeletal injuries

- Property / Equipment Damage

- Site Conditions

Job Steps

- DCP Only, Refer to operator’s manual.

- Site Hazard Assessment

- Pretrip Roller

- Fasten Seatbelt

- Start Engine

- Proceed to move roller around area to be compacted by moving the forward reverse lever in the desired direction, the further you push or pull this lever the faster the travel speed will be

- Always travel up & down slopes, not across or roll over may occur

- If full vibration compaction is required turn the vibration function on once roller is moving

- At the end of the compaction run move the forward reverse lever in the opposite direction or neutral to stop. If you choose to stop, ensure you turn the vibration function off

- Park roller in a safe place & engage the park brake