| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Peter Stead Debbie Dauphinee Person 3 | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Sidewalk Plow | Salt/Sand | Steel toe Footwear |
|  |  | Safety Vest |
|  |  | Warm Clothing |

General

To ensure the health and safety of its employees and the public, by increasing awareness of the risks associated with operating a sidewalk plow and reducing the number of accidents.  Operation of the sidewalk plow must be performed according to all relevant sections of the Motor Vehicle Act, company procedures and /or SWP’s, and manufacturer’s recommended operating guidelines.

Hazards

- Weather conditions

- Public / other vehicle

- obstacles [man made or natural

- distraction

Job Steps

Note- when starting, be certain the gear shift lever is in neutral. Next, the engine is protected by a safety shut-down system. A pair of gauges located on the oil tank above the engine, monitor oil pressure and water temperature directly. If the oil pressure drops too low or the water temperature rises too high, the system shuts of the fuel supply and the engine stops. In order to start the engine, a safety start button is located in the overhead console. Pushing this button overrides the system to allow the engine to start with no oil pressure reading on the gauge

- DCP Only, Refer to Operators Manual.

- Engage parking brake.

- Set gear change lever to neutral.

- Push throttle lever forward approx. 1”

- With left hand, push in the safety start button on the left side of the switch console and keep it pushed in while starting.

- Turn key to spring loaded position between “on” and “start” to activate the glow plugs. Hold for 30 seconds.

- While keeping the right foot off the control pedal, engage the starter with the ignition key.

- When the engine starts, release the key, which automatically returns to the “on” position and continue holding in the safety start button until the oil pressure reading rises. Then release safety start button.

- With the left foot on the brake pedal release the parking brake.

- With the engine running and the gear change lever in neutral, push the control pedal forward to start warming up the oil in the drive system.

Operation of tractor and V plow:

- While driving the tractor to first site use high range

- When plowing commences, use low range for power and safety.

- The leading edge of the plow is tapered to ride over the sidewalk cracks but the plow must be lifted to clear curbs.

- The “front lift” control lever should be placed in “float” position for normal work. Down pressure will result in less traction and front wheels and should be avoided

Operation of tractor and angle blade:

1.  The adjustable skid shoes on each end of the blade are to be set so that the blade is level and the cutting edge is at the desired height.

2. On smooth paved surfaces you may run the cutting edge in contact with the surface, while on gravel or sidewalks the uneven sections it is preferable to lower the skid shoes and leave a small amount of snow rather than continually tripping the blade.

3.When an obstacle is encountered by the cutting edge, the top of the blade trips forward and rides over the obstruction. Lift the blade and it will return to its original position.

4. Hydraulic angling is accomplished by using the RED ACC. Lever

5. A small amount of down pressure is sometimes required for hard packed snow

6.Avoid hitting vertical obstructions with the top end of the plow since it cannot trip.

7. A cross over relief valve has been installed to absorb some of the shock resulting from the end of the plow striking an object. The relief valve is adjustable to suit the conditions

Operation of the snow blower:

1.Final adjustment of the top link should be done on a level surface. By shortening the top link, the blade height is raised. No more clearance than necessary should be allowed under the cutting edge. In normal operation, it should be on the pavement.

2.The yellow ACC lever controls the chute deflector angle and the Red ACC lever controls the turning chute.

3. Snow blowing MUST always be done in low range.

4. Always engage the PTO clutch at low engine RPM.

5. Advance throttle to 2200RPM

6. The foot control pedal to the hydrostatic transmission gives the operator accurate control of forward and reverse speeds.