| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Eugene Wentzell | WAGPI JOHS Committee | 01/22/2004 |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Chains |  | Steel toe footwear |
| Binders |  | Hard Hat |
| Straps |  | Gloves |
|  |  | Safety Vest |

Job Steps

LOADING

- Park on firm level bottom and set park brake.

- Lower ramps (if trailer has any), and block rear of trailer if required.

- Load machine slowly and steadily, keeping it centered on trailer.

- Chain machine down using “Transport 70” grade chain and binders.  Use chains of a size and number to support static weight of machine.

- Load ramps and remove blocking.

- Attach any required warning devices to truck, machine or trailer.

UNLOADING

1.  Park on firm level bottom and set park brake.

2.  Reverse steps 4 and 6 above, then perform steps 2 and 3.