| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Debbie Dauphinee Blair Rawding | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
|  |  | Steel toe footwear |
|  |  | Fall Arrest Equipment |

General

Note that all work performed more than 3 metres above the nearest safe surface, or above a surface or thing that could cause injury to a person upon contact, requires the use of fall protection. This may be a fall arrest system which includes a full body harness, lanyard and anchor point; a guardrail, a safety net; temporary flooring; or a means of fall protection that provides a level of safety equal to or greater than a fall arrest system (Fall Protection Regulations s).

Job Steps

- DCP Only, Refer to Fall Protection Regulations.

- Before work starts, review the tasks to be completed and the fall protection system(s) to be used.

- Every employee who must use a fall arrest system shall be trained in its use.

- Ensure that all necessary anchor points, lifelines and static lines are in place.

- Each component of a fall arrest system, including each lifeline, shall be inspected by a competent person prior to each use.

- “Recovery Plan” must be developed to be used if an employee protected by the fall arrest system should fall and be suspended by the fall arrest system.