Safe Work Practice

EXCAVATOR

General – Only trained and experienced employees will operate an excavator

Hazards – Severe bodily impact

Musculoskeletal injuries

Fatigue

People

Other machinery and equipment

Terrain

Traffic

Complete a hazard assessment before any work begins. Know the location of buried cables. Mark them clearly before digging.

PPE – Steel toed footwear

Safety Vest

CAUTION

Whenever the tracks of the machine raise off the ground while digging, lower the machine back to the ground smoothly. DO NOT DROP OR CATCH IT WITH THE HYDRAULICS. Damage to the machine can result.

An oversized bucket or one equipped with blade-type side cutters, should not be used in rocky material. It slows down the cycle and damage to the bucket and other machine components could result.

Use smooth comfortable speeds while operating For efficient operation, use more than one control  at a time when possible.

Spot a truck so the machine can load it from the rear or the side. Load trucks evenly to avoid overloading rear axles. Do not swing over the cab of a truck.

WARNING .

When moving with a clamshell equipped machine, keep the stick extended to prevent the clamshell from swinging into the cab.

Move the machine whenever the digging position is not efficient. The machine can be moved forward or  backward anytime during the operating cycle.

Utilize the bucket or other attachment to push or pull the machine or to lift the tracks, in order to work in and out of close places.

When making turns in soft material, travel in a straight direction occasionally to clear the tracks.

EXCAVATOR

When swinging into a ditch, do not use the ditch to stop the swinging motion.  Inspect the machine for damage if the boom is swung into a bank or an object.

Repeated chopping can cause structural damage. Use a digging, scratching or peeling motion when trying to penetrate rock, frozen ground or hard surfaces.

With certain boom-stick-bucket combinations, the bucket can hit the cab or front of the machine.  Always check for interference when first operating a new attachment

1.  With the governor control in the operating range  position the stick at a 70º angle to the ground.

2.  Position the bucket cutting edge at a 120º angle to the ground

3. Move the stick in and keep the bucket parallel

4.  If stick travel stops due to the load, adjust the ground depth of cut by raising the boom and / or curling the bucket

Use the hook on the back of the bucket to sling objects. Lifting capabilities are calculated from this point

An unstable condition can exist if a load exceeds the machine load rating or if a heavy load is swung

over an end or side.

Picking up a load over a corner is the most stable position.

For the best stability, carry a load close to the ground.

Lifting capacity decreases as the distance of the load from the swing bearing centerline is increased.

5. To apply the greatest force at the cutting edge, down pressure must be decreased as the stick moves in.

6. Maintain a bucket attitude that assures a continuous flow of material into the bucket.

7. Continue the pass with the bucket traveling in a horizontal direction, peeling material into it.

8. Engage the swing control when the bucket is clear of the excavation.

9. To dump, move the stick out and open the bucket in one smooth motion. To provide a working surface, dump the spoil away from the edge of a trench.

A WARNING

Do not exceed the rated load capacity of the machine. If the machine is not on level ground, load capacities will vary.

Use short slings to prevent excessive load swing.

CAUTION

Damage to bucket cylinder, bucket or linkage could result if slings are placed incorrectly