Safe Work Practice

Vehicle Oil Change

General- Oil changes are an important maintenance step for a vehicle and will extend the life of the vehicle.  Oil changes are to be performed by a competent person using all necessary safety measures. Using the incorrect oil may damage the engine. Proper disposal is also required to keep environmental exposure limited.

Hazards-Vehicle damage

Eye injury

Severe bodily impact

Musculoskeletal injuries

Environmental damage

Fire

Explosion

Burns

Toxic exposure

PPE- Steel toed footwear

Eye Protection

Gloves

Coveralls

- Inspect work area to ensure that it is free of hazards. Read appropriate MSDS prior to starting oil change

- If vehicle is being raised, follow procedure appropriate to the equipment being used. If vehicle is to be jacked up refer to vehicle service manual for placement of jack and safety stands. If a hoist is being used refer to manual for proper instruction for that type vehicle.

- Loosen but do not remove drain plug. Place waste oil recovery tank or container as close as possible to oil drain and then remove the plug. When you remove the filter , there will also be oil draining or dripping from that location. If there is a large oil spill, clean it up while oil is draining.

- Replace drain plug and new oil filter following the manufacturers instructions/recommendations. [Remember to always hand tighten the oil filter.DO NOT USE ANY TOOLS TO TIGHTEN THE OIL FILTER] Install new oil into vehicle per vehicle service manual. Check under vehicle to make sure oil is not leaking out. Clean area of spills and recycle old oil and oil filter in appropriate storage area