Safe Work Practice

Working Around Heavy Equipment

General – Heavy equipment is a necessity on nearly every construction project. Unfortunately, they can be extremely dangerous when not used properly. Rollovers, struck by accidents and caught in or between accidents are common ways workers can be injured when working on or near heavy equipment.

Hazards – severe bodily injury

musculoskeletal injuries

shock

terrain

PPE – steel toed footwear

safety vest

hard hat

ear protection

-Have a defined route and clear visual path for the operator when moving equipment from the point of origin to the work site.

-Set up of the equipment at the work area should be stable and have enough space to allow the equipment and workers to perform the planed tasks.

-Use 3 point mounting and dismounting technique off of heavy equipment - NEVER JUMP OFF HEAVY EQUIPMENT.

-Establish a danger zone, that is; the working area where contact could result in personal injury or damage during operations.

-Predefined hand signals or use of two way radios between the operator and person in charge of the work crew to accomplish any and all movement.

-Maintain a clear line of site between the operator and workers.  Blind spots are common. If you can’t see the operator, they can’t see you.

-Use a spotter when heavy equipment is in motion.  This requires communication between the operator and workers to maintain safe movement.

-Always try to walk on the driver side of equipment as the passenger side has a larger blind spot.

-Workers should keep a safe distance from all sides of the heavy equipment while it is in use.

-Be aware of the swing radius on certain equipment and, if possible, cordon off the area with barriers or caution tape.

-Wear high visibility clothing and Personal Protective Equipment

Many times when workers think they can be seen by the operators of heavy equipment they can’t.  You may be in a blind spot or the operator has been driving is a designated travel path for a period of time and the only thing that has changed is you. Always stay alert