Safe Work Practice

Working with Concrete/Cement

General-Concrete can also cause chemical burns, so be sure to wear the proper protective clothing, such as long sleeves, pants, waterproof gloves, protective safety goggles, and high rubber boots. You'll also want to wear goggles and a face mask when sanding concrete to prevent inhaling concrete dust.

PPE – Steel toed footwear

Eye Protection

Ear protection

Respirator

Long sleeves

Hazards-Slips, trips & falls

Explosive dust

Inhalation of dust particles

Flying debris

Skin irritation

Practices

- • DO NOT have prolonged contact with concrete, it can cause respiratory illness, burns, rashes and skin irritation. - • Ensure appropriate PPE is used. Wet concrete can cause severe burns to the skin. If wet concrete gets on the skin it should be washed off completely as soon as contact occurs. - • Become familiar with the Safety Data Sheet for the concrete/cement products you are using to ensure you know the proper personal protective equipment to use and what to do if exposure occurs. - • Avoid breathing concrete dust by: o Avoiding dusty areas - o Wetting down the work area to minimize dust - o Wearing approved respiratory protection - o Use HEPA vacuums to collect dust, don’t dry sweep. - o Using wet cuts, rather than dry cuts. - o Mix dry cement in well ventilated areas. - o Work upwind from dust sources.